DMX Channel Sheet

- from software version 1.61

- (Instruction version 1.55)

e-mail: service@glp.de

Internet: http://www.glp.de

DMX Channels

| Normal-Mode 24 DMX Channels |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel | Function | Time and Value | DMX |  | HEX |  | % |  |
| 1) PAN- coarse | 0 .. 660° |  | 0..255 |  | 00..FF |  | 0..100 |  |
| 2) PAN-fine | High- Pos ... High- Pos + 2,6° (16 Bit) |  | 0..255 |  | 00..FF |  | 0..100 |  |
| 3) Tilt- coarse | 0 .. 280° |  | 0..255 |  | 00..FF |  | 0..100 |  |
| 4) Tilt-fine | High- Pos … High- Pos + 1,2° (16 Bit) |  | 0..255 |  | 00..FF |  | 0..100 |  |
| 5) Color | Colors adjustable via RGB |  | 0..7 |  | 00..07 |  | 0..2,5 |  |
| (fixed) | Color 01 - Red |  | 8..15 |  | 08..0F |  | 3..5,5 |  |
|  | Color 02 - Amber |  | 16..23 |  | 10..17 |  | 6..8,5 |  |
|  | Color 03 – Yellow warm |  | 24..31 |  | 18..1F |  | 9..12,5 |  |
|  | Color 04 - Yellow |  | 32..39 |  | 20..27 |  | 13..15,5 |  |
|  | Color 05 – Green |  | 40..47 |  | 28..2F |  | 16..18,5 |  |
|  | Color 06 – Turquoise |  | 48..55 |  | 30..37 |  | 19..21,5 |  |
|  | Color 07 – Cyan |  | 56..63 |  | 38..3F |  | 22..24,5 |  |
|  | Color 08 - Blue |  | 64..71 |  | 40..47 |  | 25..27,5 |  |
|  | Color 09 - Lavender |  | 72..79 |  | 48..4F |  | 28..30,5 |  |
|  | Color 10 - Malve |  | 80..87 |  | 50..57 |  | 31..34,5 |  |
|  | Color 11 – Magenta |  | 88..95 |  | 58..5F |  | 35..37,5 |  |
|  | Color 12 - Pink |  | 96..103 |  | 60..67 |  | 38..40,5 |  |
|  | White - CTO | Color temp. 3200K | 104..111 |  | 68..6F |  | 41..43,5 |  |
|  | White | Color temp. 5600K | 112..119 |  | 70..77 |  | 44..46,5 |  |
|  | White – CTB | Color temp. 7200K | 120..127 |  | 78..7F |  | 47..49,5 |  |
|  | Rainbow Effect Stop | Stop | 128 |  | 80 |  | 50 |  |
|  | Rainbow Effect | slow – fast Color 01 -> Color 12 | 129..255 |  | 81..FF |  | 51..100 |  |
| 6) Red | Color mixing system - Red | 0 - 100% | 0..255 |  | 00..FF |  | 0..100 |  |
| 7) Green | Color mixing system - Green | 0 - 100% | 0..255 |  | 00..FF |  | 0..100 |  |
| 8) Blue | Color mixing system - Blue | 0 - 100% | 0..255 |  | 00..FF |  | 0..100 |  |
| 9) Shutter | Shutter closed |  | 0..15 |  | 00..0F |  | 0..5,5 |  |
|  | Shutter pulse random | slow – fast | 16..47 |  | 10..2F |  | 6..18,5 |  |
|  | Shutter ramp up | slow – fast | 48..79 |  | 30..4F |  | 19..31 |  |
|  | Shutter ramp down | slow – fast | 80..111 |  | 50..6F |  | 32..43 |  |
|  | Shutter ramp up/down | slow – fast | 112..143 |  | 70..8F |  | 44..56 |  |
|  | Random strobe | slow – fast | 144..159 |  | 90..9F |  | 57..62 |  |
|  | Strobe effect pause | 5s …0,1s | 160..199 |  | A0..C7 |  | 63..77 |  |
|  | Strobe effect slow - fast | 1 Hz .. 25 Hz | 200..241 |  | C8..F1 |  | 78..95 |  |
|  | Shutter open |  | 242..255 |  | F2..FF |  | 96..100 |  |
| 10) Dimmer | Dimmer | 0% - 100% | 0..255 |  | 0..FF |  | 0..100 |  |
| 11) Color temp. | No color temperature correction |  | 0..7 |  | 0..07 |  | 0..2,5 |  |
|  | Continuous color temperature correction between 7200k - 3200k | Applicable for ALL colors 5) | 8..255 |  | 08..FF |  | 3..100 |  |
| 12) Gobo Shake & Special | Gobo Shake for Gobowheel 1+2 off |  | 0 .. 15 |  | 00..0F |  | 0..5,5 |  |
|  | Gobo Shake Gobowheel 1 | Speed 1 Size 1..8 | 16..23 |  | 10..17 |  | 6..9 |  |
|  |  | Speed 2 Size 1..8 | 24..31 |  | 18..1F |  | 9,5..12 |  |
|  |  | Speed 3 Size 1..8 | 32..39 |  | 20..27 |  | 12,5..15 |  |
|  |  | Speed 4 Size 1..8 | 40..47 |  | 28..2F |  | 15,5..18 |  |
|  |  | Speed 5 Size 1..8 | 48..55 |  | 30..37 |  | 18,5..21,5 |  |
|  |  | Speed 6 Size 1..8 | 56..63 |  | 38..3F |  | 22..24,5 |  |
|  |  | Speed 7 Size 1..8 | 64..71 |  | 40..47 |  | 25..27,5 |  |
|  |  | Speed 8 Size 1..8 | 72..79 |  | 48..4F |  | 28..30,5 |  |
|  | Gobo Shake Gobowheel 2 | Speed / Size see Gobo Shake Gobowheel 1 | 80..143 |  | 50..90 |  | 31..56 |  |
|  | Gobo Shake Gobowheel 1+2 | Speed / Size see Gobo Shake Gobowheel 2 | 144..207 |  | 91..CF |  | 56,5..81 |  |
|  | Reset 1 | Only Head | 250..253 |  | FA..FC |  | 98..99 |  |
|  | Reset 2 | All Funktions | 254..255 |  | FD..FF |  | 100 |  |
| 13) Movement | No movement |  | 0 |  | 0 |  | 0 |  |
|  | Movement | Size | Phase |  |  |  |  |  |
|  | PAN | 1 | 0° |  |  |  | 01..01 |  |
|  |  | 1 | 90° |  |  |  | 02..03 |  |
|  |  | 1 | 180° |  |  |  | 04..05 |  |
|  |  | 1 | 270° |  |  |  | 06..07 |  |
|  | PAN | 2 | 0° |  |  |  | 08..09 |  |
|  |  | 2 | 90° |  |  |  | 10..11 |  |
|  |  | 2 | 180° |  |  |  | 12..13 |  |
|  |  | 2 | 270° |  |  |  | 14..15 |  |
|  | PAN | 3 | 0° |  |  |  | 16..17 |  |
|  |  | 3 | 90° |  |  |  | 18..19 |  |
|  |  | 3 | 180° |  |  |  | 20..21 |  |
|  |  | 3 | 270° |  |  |  | 22..23 |  |
|  | PAN | 4 | 0° |  |  |  | 24..25 |  |
|  |  | 4 | 90° |  |  |  | 26..27 |  |
|  |  | 4 | 180° |  |  |  | 28..29 |  |
|  |  | 4 | 270° |  |  |  | 30..31 |  |
|  | TILT | size / phase see also PAN | 32..63 |  | 20..3F |  | 13..25 |  |
|  | PAN / TILT | size / phase see also PAN | 64..95 |  | 40..5F |  | 26..37 |  |
|  | PAN / TILT (inverse) | size / phase see also PAN | 96..127 |  | 60..7F |  | 38..50 |  |
|  | Circle | size / phase see also PAN | 128..159 |  | 80..9F |  | 51..62 |  |
|  | Circle (inverse) | size / phase see also PAN | 160..191 |  | A0..BF |  | 63..75 |  |
|  | Lying eight | size / phase see also PAN | 192..223 |  | C0..DF |  | 76..87 |  |
|  | Random movement | size see also PAN | 224..255 |  | E0..FF |  | 88..100 |  |
| 14) Speed | Pan/Tilt relative movement |  | 0..1 |  | 00..01 |  | 0..0,5 |  |
| Pan/Tilt | Pan/Tilt slow – fast Use this channel 14) also for the speed of the movements (channel 13). | Pan Min. 660° Pan Max. 660° Tilt Min. 280° Tilt Max. 280° | 2..255 |  | 02..FF |  | 1..100 |  |
| 15) Zoom | Stepless adjustable Zoom | 10° – 32° | 0 .. 255 |  | 0..FF |  | 0..100 |  |
| 16) Focus | Stepless adjustable Focus | infinity – near | 0 .. 255 |  | 0..FF |  | 0..100 |  |
| 17) Gobo1 | Gobo 1 (open,) |  | 0…15 |  | 00..0F |  | 0..5,9 |  |
| (indexed) |  |  |  |  |  |  |  |  |
|  | Gobo 2 |  | 16..31 |  | 10..1F |  | 6..11,9 |  |
|  | Gobo 3 |  | 32..47 |  | 20.. 2F |  | 12..17,9 |  |
|  | Gobo 4 |  | 48..63 |  | 30.. 3F |  | 19.. 24,5 |  |
|  | Gobo 5 |  | 64..79 |  | 40..4F |  | 25..30,5 |  |
|  | Gobo 6 |  | 80..95 |  | 50..5F |  | 31..37,5 |  |
|  | Gobo |  | 96..111 |  | 60..6F |  | 38..43,5 |  |
|  | Gobo 8 |  | 112..126 |  | 70..82 |  | 44..48 |  |
|  | Gobo rotation STOP |  | 127 |  | 7F |  | 49 |  |
|  | Gobo rotation, slow-fast, CW |  | 128..192 |  | 80..C0 |  | 50..76 |  |
|  | Gobo rotation, fast-slow, CCW |  | 193..255 |  | C1..FF |  | 77..100 |  |
| 18) Gobo 1 | Gobo position 0 ... 540° |  | 0..127 |  | 00..7F |  | 0..49 |  |
| Posi./Rot | Gobo rotation STOP |  | 128..129 |  | 80..81 |  | 50 |  |
|  | Gobo rotation, slow-fast, CW |  | 130..192 |  | 82..C0 |  | 51..75 |  |
|  | Gobo rotation, fast-slow, CCW |  | 193..253 |  | C1..FD |  | 76..99 |  |
|  | Gobo rotation STOP |  | 254..255 |  | FD..FF |  | 99..100 |  |
| 19)Gobo2 | Gobo 1 (open) |  | 0…15 |  | 00..0F |  | 0..5,9 |  |
| (indexed) |  |  |  |  |  |  |  |  |
|  | Gobo 2 |  | 16..31 |  | 10..1F |  | 6..11,9 |  |
|  | Gobo 3 |  | 32..47 |  | 20.. 2F |  | 12..17,9 |  |
|  | Gobo 4 |  | 48..63 |  | 30.. 3F |  | 19.. 24,5 |  |
|  | Gobo 5 |  | 64..79 |  | 40..4F |  | 25..30,5 |  |
|  | Gobo 6 |  | 80..95 |  | 50..5F |  | 31..37,5 |  |
|  | Gobo 7 |  | 96..111 |  | 60..6F |  | 38..43,5 |  |
|  | Gobo 8 |  | 112..126 |  | 70..82 |  | 44..48 |  |
|  | Gobo rotation STOP |  | 127 |  | 7F |  | 49 |  |
|  | Gobo rotation, slow-fast, CW |  | 128..192 |  | 80..C0 |  | 50..76 |  |
|  | Gobo rotation, fast-slow, CCW |  | 193..255 |  | C1..FF |  | 77..100 |  |
| 20) Gobo 2 | Gobo position 0 ... 540° |  | 0..127 |  | 00..7F |  | 0..49 |  |
| Posi./Rot | Gobo rotation STOP |  | 128..129 |  | 80..81 |  | 50 |  |
|  | Gobo rotation, slow-fast, CW |  | 130..192 |  | 82..C0 |  | 51..75 |  |
|  | Gobo rotation, fast-slow, CCW |  | 193..253 |  | C1..FD |  | 76..99 |  |
|  | Gobo rotation STOP |  | 254..255 |  | FD..FF |  | 99..100 |  |
| 21) Effect wheel swing | Stepless swing of the "Motion Wheel" | 0 - 100% | 0..255 |  | 00..FF |  | 0..100 |  |
|  | Rotating stop |  | 0..1 |  | 00..01 |  | 00..1 |  |
| 22) Effect wheel | Rotating slow - fast CW |  | 1..128 |  | 01..80 |  | 1..50 |  |
| Posi./Rot | Rotating fast - slow CCW |  | 129...254 |  | 81..FD |  | 51..99 |  |
|  | Rotating stop |  | 254..255 |  | FD..FF |  | 99..100 |  |
| 23) Prism | Prism swing out |  | 0..5 |  | 00..05 |  | 0..2 |  |
|  | Prism position 0 ... 540° |  | 6..127 |  | 06..7F |  | 0..50 |  |
|  | Prism rotation stop |  | 128..129 |  | 80..81 |  | 50,1 |  |
|  | Prism rotation, slow-fast, CW |  | 130..192 |  | 82..C0 |  | 51..75 |  |
|  | Prism rotation, fast-slow, CCW |  | 193..253 |  | C1..FD |  | 76..99 |  |
|  | Prism rotation stop |  | 254..255 |  | FD..FF |  | 99.100 |  |
| 24) Iris | Iris open – closed |  | 0..131 |  | 00..83 |  | 00..52 |  |
| Compress-Mode 20 DMX Channels |  |  |  |  |  |  |  |  |
| Channel | Function | Time and Value | DMX | HEX |  | % |  |  |
| 1) PAN- coarse | 0 .. 660° |  | 0..255 | 00..FF |  | 0..100 |  |  |
| 2) PAN-fine | High- Pos ... High- Pos +(16 Bit) |  | 0..255 | 00..FF |  | 0..100 |  |  |
| 3) Tilt- coarse | 0 .. 280° |  | 0..255 | 00..FF |  | 0..100 |  |  |
| 4) Tilt-fine | High- Pos … High- Pos + (16 Bit) |  | 0..255 | 00..FF |  | 0..100 |  |  |
| 5) Color | Colors adjustable via RGB |  | 0..7 | 00..07 |  | 0..2,5 |  |  |
| (fixed) | Color 01 - Red |  | 8..15 | 08..0F |  | 3..5,5 |  |  |
|  | Color 02 - Amber |  | 16..23 | 10..17 |  | 6..8,5 |  |  |
|  | Color 03 – Yellow warm |  | 24..31 | 18..1F |  | 9..12,5 |  |  |
|  | Color 04 - Yellow |  | 32..39 | 20..27 |  | 13..15,5 |  |  |
|  | Color 05 – Green |  | 40..47 | 28..2F |  | 16..18,5 |  |  |
|  | Color 06 – Turquoise |  | 48..55 | 30..37 |  | 19..21,5 |  |  |
|  | Color 07 – Cyan |  | 56..63 | 38..3F |  | 22..24,5 |  |  |
|  | Color 08 - Blue |  | 64..71 | 40..47 |  | 25..27,5 |  |  |
|  | Color 09 - Lavender |  | 72..79 | 48..4F |  | 28..30,5 |  |  |
|  | Color 10 - Malve |  | 80..87 | 50..57 |  | 31..34,5 |  |  |
|  | Color 11 – Magenta |  | 88..95 | 58..5F |  | 35..37,5 |  |  |
|  | Color 12 - Pink |  | 96..103 | 60..67 |  | 38..40,5 |  |  |
|  | White - CTO | Color temp. 3200K | 104..111 | 68..6F |  | 41..43,5 |  |  |
|  | White | Color temp. 5600K | 112..119 | 70..77 |  | 44..46,5 |  |  |
|  | White – CTB | Color temp. 7200K | 120..127 | 78..7F |  | 47..49,5 |  |  |
|  | Rainbow Effect Stop |  | 128 | 80 |  | 50 |  |  |
|  | Rainbow Effect | slow – fast Color 01 -> Color 12 | 129..255 | 81..FF |  | 51..100 |  |  |
| 6) Red | Color mixing system - Red | 0 - 100% | 0..255 | 00..FF |  | 0..100 |  |  |
| 7) Green | Color mixing system - Green | 0 - 100% | 0..255 | 00..FF |  | 0..100 |  |  |
| 8) Blue | Color mixing system - Blue | 0 - 100% | 0..255 | 00..FF |  | 0..100 |  |  |
| 9) Shutter | Shutter closed |  | 0..15 | 00..0F |  | 0..5,5 |  |  |
|  | Shutter pulse random | slow – fast | 16..47 | 10..2F |  | 6..18,5 |  |  |
|  | Shutter ramp up | slow – fast | 48..79 | 30..4F |  | 19..31 |  |  |
|  | Shutter ramp down | slow – fast | 80..111 | 50..6F |  | 32..43 |  |  |
|  | Shutter ramp up/down | slow – fast | 112..143 | 70..8F |  | 44..56 |  |  |
|  | Random strobe | slow – fast | 144..159 | 90..9F |  | 57..62 |  |  |
|  | Strobe effect pause | 5s …0,1s | 160..199 | A0..C7 |  | 63..77 |  |  |
|  | Strobe effect slow - fast | 1 Hz .. 25 Hz | 200..241 | C8..F1 |  | 78..95 |  |  |
|  | Shutter open |  | 242..249 | F2..F9 |  | 95,5..96,5 |  |  |
|  | RESET 1 | Only Head | 250 | FA |  | 97 |  |  |
|  | RESET 2 | All Funktions | 251 | FB |  | 98 |  |  |
|  | Shutter open |  | 252..255 | FC..FF |  | 99..100 |  |  |
| 10) Dimmer | Dimmer | 0% - 100% | 0..255 | 0..FF |  | 0..100 |  |  |
| 11) Zoom | Stepless adjustable Zoom | 10° – 32° | 0 .. 255 | 0..FF |  | 0..100 |  |  |
| 12) Focus | Stepless adjustable Focus | infinity – near | 0 .. 255 | 0..FF |  | 0..100 |  |  |
| 13)Gobo1 | Gobo 1 (open) |  | 0…15 | 00..0F |  | 0..5,9 |  |  |
| (indexed) |  |  |  |  |  |  |  |  |
|  | Gobo 2 |  | 16..31 | 10..1F |  | 6..11,9 |  |  |
|  | Gobo 3 |  | 32..47 | 20.. 2F |  | 12..17,9 |  |  |
|  | Gobo 4 |  | 48..63 | 30.. 3F |  | 19.. 24,5 |  |  |
|  | Gobo 5 |  | 64..79 | 40..4F |  | 25..30,5 |  |  |
|  | Gobo 6 |  | 80..95 | 50..5F |  | 31..37,5 |  |  |
|  | Gobo |  | 96..111 | 60..6F |  | 38..43,5 |  |  |
|  | Gobo 8 |  | 112..126 | 70..82 |  | 44..48 |  |  |
|  | Gobo rotation STOP |  | 127 | 7F |  | 49 |  |  |
|  | Gobo rotation, slow-fast, CW |  | 128..192 | 80..C0 |  | 50..76 |  |  |
|  | Gobo rotation, fast-slow, CCW |  | 193..255 | C1..FF |  | 77..100 |  |  |
| 14) Gobo 1 | Gobo position 0 ... 540° |  | 0..127 | 00..7F |  | 0..49 |  |  |
| Posi./Rot | Gobo rotation STOP |  | 128..129 | 80..81 |  | 50 |  |  |
|  | Gobo rotation, slow-fast, CW |  | 130..192 | 82..C0 |  | 51..75 |  |  |
|  | Gobo rotation, fast-slow, CCW |  | 193..253 | C1..FD |  | 76..99 |  |  |
|  | Gobo rotation STOP |  | 254..255 | FD..FF |  | 99..100 |  |  |
| 15)Gobo2 | Gobo 1 (open, fast) |  | 0…15 | 00..0F |  | 0..5,9 |  |  |
| (indexed) |  |  |  |  |  |  |  |  |
|  | Gobo 2 |  | 16..31 | 10..1F |  | 6..11,9 |  |  |
|  | Gobo 3 |  | 32..47 | 20.. 2F |  | 12..17,9 |  |  |
|  | Gobo 4 |  | 48..63 | 30.. 3F |  | 19.. 24,5 |  |  |
|  | Gobo 5 |  | 64..79 | 40..4F |  | 25..30,5 |  |  |
|  | Gobo 6 |  | 80..95 | 50..5F |  | 31..37,5 |  |  |
|  | Gobo 7 |  | 96..111 | 60..6F |  | 38..43,5 |  |  |
|  | Gobo 8 |  | 112..126 | 70..82 |  | 44..48 |  |  |
|  | Gobo rotation STOP |  | 127 | 7F |  | 49 |  |  |
|  | Gobo rotation, slow-fast, CW |  | 128..192 | 80..C0 |  | 50..76 |  |  |
|  | Gobo rotation, fast-slow, CCW |  | 193..255 | C1..FF |  | 77..100 |  |  |
| 16) Gobo 2 | Gobo position 0 ... 540° |  | 0..127 | 00..7F |  | 0..49 |  |  |
| Posi./Rot | Gobo rotation STOP |  | 128..129 | 80..81 |  | 50 |  |  |
|  | Gobo rotation, slow-fast, CW |  | 130..192 | 82..C0 |  | 51..75 |  |  |
|  | Gobo rotation, fast-slow, CCW |  | 193..253 | C1..FD |  | 76..99 |  |  |
|  | Gobo rotation STOP |  | 254..255 | FD..FF |  | 99..100 |  |  |
| 17) Effect wheel swing | Stepless swing of the "Motion Wheel" | 0 - 100% | 0..255 | 00..FF |  | 0..100 |  |  |
|  | Rotating stop |  | 0..1 | 00..01 |  | 00..1 |  |  |
| 18) Effect wheel | Rotating slow - fast CW |  | 1..128 | 01..80 |  | 1..50 |  |  |
| Posi./Rot | Rotating fast - slow CCW |  | 129...254 | 81..FD |  | 51..99 |  |  |
|  | Rotating stop |  | 254..255 | FD..FF |  | 99..100 |  |  |
| 19) Prism | Prism swing out |  | 0..5 | 00..05 |  | 0..2 |  |  |
|  | Prism position 0 ... 540° |  | 6..127 | 06..7F |  | 0..50 |  |  |
|  | Prism rotation stop |  | 128..129 | 80..81 |  | 50,1 |  |  |
|  | Prism rotation, slow-fast, CW |  | 130..192 | 82..C0 |  | 51..75 |  |  |
|  | Prism rotation, fast-slow, CCW |  | 193..253 | C1..FD |  | 76..99 |  |  |
|  | Prism rotation stop |  | 254..255 | FD..FF |  | 99.100 |  |  |
| 20) Iris | Iris open – closed |  | 0..131 | 00..83 |  | 00..52 |  |  |

| High Resolution (Extended)-Mode 31 DMX Channels |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Kanal | Funktion | Zeiten und Werte | DMX | HEX | % |
| 1) PAN- coarse | 0 .. 660° |  | 0..255 | 00..FF | 0..100 |
| 2) PAN-fine | High- Pos ... High- Pos + (16 Bit) |  | 0..255 | 00..FF | 0..100 |
| 3) Tilt- coarse | 0 .. 280° |  | 0..255 | 00..FF | 0..100 |
| 4) Tilt-fine | High- Pos … High- Pos + (16 Bit) |  | 0..255 | 00..FF | 0..100 |
| 5) Color | Colors adjustable via RGB |  | 0..7 | 00..07 | 0..2,5 |
| (fixed) | Color 01 - Red |  | 8..15 | 08..0F | 3..5,5 |
|  | Color 02 - Amber |  | 16..23 | 10..17 | 6..8,5 |
|  | Color 03 – Yellow warm |  | 24..31 | 18..1F | 9..12,5 |
|  | Color 04 - Yellow |  | 32..39 | 20..27 | 13..15,5 |
|  | Color 05 – Green |  | 40..47 | 28..2F | 16..18,5 |
|  | Color 06 – Turquoise |  | 48..55 | 30..37 | 19..21,5 |
|  | Color 07 – Cyan |  | 56..63 | 38..3F | 22..24,5 |
|  | Color 08 - Blue |  | 64..71 | 40..47 | 25..27,5 |
|  | Color 09 - Lavender |  | 72..79 | 48..4F | 28..30,5 |
|  | Color 10 - Malve |  | 80..87 | 50..57 | 31..34,5 |
|  | Color 11 – Magenta |  | 88..95 | 58..5F | 35..37,5 |
|  | Color 12 - Pink |  | 96..103 | 60..67 | 38..40,5 |
|  | White - CTO | Color temp. 3200K | 104..111 | 68..6F | 41..43,5 |
|  | White | Color temp. 5600K | 112..119 | 70..77 | 44..46,5 |
|  | White – CTB | Color temp. 7200K | 120..127 | 78..7F | 47..49,5 |
|  | Rainbow Effect Stop |  | 128 | 80 | 50 |
|  | Rainbow Effect | slow – fast Color 01 ->Color 12 | 129..255 | 81..FF | 51..100 |
| 6) Red- coarse | Color mixing system – Red | 0 - 100% | 0..255 | 00..FF | 0..100 |
| 7) Red-fine | Color mixing system – Red-low |  | 0..255 | 00..FF | 0..100 |
| 8) Green- coarse | Color mixing system – Green | 0 - 100% | 0..255 | 00..FF | 0..100 |
| 9) Green- fine | Color mixing system – Green-low |  | 0..255 | 00..FF | 0..100 |
| 10) Blue- coarse | Color mixing system – Blue | 0 - 100% | 0..255 | 00..FF | 0..100 |
| 11) Blue-fine | Color mixing system – Blue-low |  | 0..255 | 00..FF | 0..100 |
| 12) Shutter | Shutter closed |  | 0..15 | 00..0F | 0..5,5 |
|  | Shutter pulse random | slow – fast | 16..47 | 10..2F | 6..18,5 |
|  | Shutter ramp up | slow – fast | 48..79 | 30..4F | 19..31 |
|  | Shutter ramp down | slow – fast | 80..111 | 50..6F | 32..43 |
|  | Shutter ramp up/down | slow – fast | 112..143 | 70..8F | 44..56 |
|  | Random strobe | slow – fast | 144..159 | 90..9F | 57..62 |
|  | Strobe effect pause | 5s …0,1s | 160..199 | A0..C7 | 63..77 |
|  | Strobe effect slow - fast | 1 Hz .. 25 Hz | 200..241 | C8..F1 | 78..95 |
|  | Shutter open |  | 242..255 | F2..FF | 96..100 |
| 13)Dimmer-coarse | Dimmer | 0% - 100% | 0..255 | 0..FF | 0..100 |
| 14) Dimmer- fine | Dimmer low |  | 0..255 | 0..FF | 0..100 |
| 15) Color temp. | No color temperature correction |  | 0..7 | 0..07 | 0..2,5 |
|  | Continuous color temperature correction between 7200k - 3200k | Applicable for ALL colors 5) | 8..255 | 08..FF | 3..100 |
| 12) Gobo Shake & Special | Gobo Shake for Gobowheel 1+2 off |  | 0 .. 15 | 00..0F | 0..5,5 |
|  | Gobo Shake Gobowheel 1 | Speed 1 Size 1..8 | 16..23 | 10..17 | 6..9 |
|  |  | Speed 2 Size 1..8 | 24..31 | 18..1F | 9,5..12 |
|  |  | Speed 3 Size 1..8 | 32..39 | 20..27 | 12,5..15 |
|  |  | Speed 4 Size 1..8 | 40..47 | 28..2F | 15,5..18 |
|  |  | Speed 5 Size 1..8 | 48..55 | 30..37 | 18,5..21,5 |
|  |  | Speed 6 Size 1..8 | 56..63 | 38..3F | 22..24,5 |
|  |  | Speed 7 Size 1..8 | 64..71 | 40..47 | 25..27,5 |
|  |  | Speed 8 Size 1..8 | 72..79 | 48..4F | 28..30,5 |
|  | Gobo Shake Gobowheel 2 | Speed / Size see Gobo Shake Gobowheel 1 | 80..143 | 50..90 | 31..56 |
|  | Gobo Shake Gobowheel 1+2 | Speed / Size see Gobo Shake Gobowheel 2 | 144..207 | 91..CF | 56,5..81 |
|  | Reset 1 | Only Head | 250..253 | FA..FC | 98..99 |
|  | Reset 2 | All Funktions | 254..255 | FD..FF | 100 |
| 17) Zoom coarse | Stepless adjustable Zoom | 10° – 32° | 0 .. 255 | 0..FF | 0..100 |
| 18) Zoom fine | Zoom low | (0% - 100%) | 0..255 | 0..FF | 0..100 |
| 19) Focus coarse | Stepless adjustable Focus | infinity – near | 0 .. 255 | 0..FF | 0..100 |
| 20) Focus fine | Focus low | (0% - 100%) | 0..255 | 0..FF | 0..100 |
| 21)Gobo1 | Gobo 1 (open) |  | 0…15 | 00..0F | 0..5,9 |
| (indexed) |  |  |  |  |  |
|  | Gobo 2 |  | 16..31 | 10..1F | 6..11,9 |
|  | Gobo 3 |  | 32..47 | 20.. 2F | 12..17,9 |
|  | Gobo 4 |  | 48..63 | 30.. 3F | 19.. 24,5 |
|  | Gobo 5 |  | 64..79 | 40..4F | 25..30,5 |
|  | Gobo 6 |  | 80..95 | 50..5F | 31..37,5 |
|  | Gobo 7 |  | 96..111 | 60..6F | 38..43,5 |
|  | Gobo 8 |  | 112..126 | 70..82 | 44..48 |
|  | Gobo rotation STOP |  | 127 | 7F | 49 |
|  | Gobo rotation, slow-fast, CW |  | 128..192 | 80..C0 | 50..76 |
|  | Gobo rotation, fast-slow, CCW |  | 193..255 | C1..FF | 77..100 |
| 22) Gobo 1 | Gobo position 0 ... 540° |  | 0..127 | 00..7F | 0..49 |
| Posi./Rot coarse | Gobo rotation STOP |  | 128..129 | 80..81 | 50 |
|  | Gobo rotation, slow-fast, CW |  | 130..192 | 82..C0 | 51..75 |
|  | Gobo rotation, fast-slow, CCW |  | 193..253 | C1..FD | 76..99 |
|  | Gobo rotation STOP |  | 254..255 | FD..FF | 99..100 |
| 23) Gobo 1 rot fine | Gobo 1 rot. low | (0% - 100%) | 0..255 | 0..FF | 0..100 |
| 24)Gobo2 | Gobo 1 (open) |  | 0…15 | 00..0F | 0..5,9 |
| (indexed) |  |  |  |  |  |
|  | Gobo 2 |  | 16..31 | 10..1F | 6..11,9 |
|  | Gobo 3 |  | 32..47 | 20.. 2F | 12..17,9 |
|  | Gobo 4 |  | 48..63 | 30.. 3F | 19.. 24,5 |
|  | Gobo 5 |  | 64..79 | 40..4F | 25..30,5 |
|  | Gobo 6 |  | 80..95 | 50..5F | 31..37,5 |
|  | Gobo 7 |  | 96..111 | 60..6F | 38..43,5 |
|  | Gobo 8 |  | 112..126 | 70..82 | 44..48 |
|  | Gobo rotation STOP |  | 127 | 7F | 49 |
|  | Gobo rotation, slow-fast, CW |  | 128..192 | 80..C0 | 50..76 |
|  | Gobo rotation, fast-slow, CCW |  | 193..255 | C1..FF | 77..100 |
| 25) Gobo 2 | Gobo position 0 ... 540° |  | 0..127 | 00..7F | 0..49 |
| Posi./Rot coarse | Gobo rotation STOP |  | 128..129 | 80..81 | 50 |
|  | Gobo rotation, slow-fast, CW |  | 130..192 | 82..C0 | 51..75 |
|  | Gobo rotation, fast-slow, CCW |  | 193..253 | C1..FD | 76..99 |
|  | Gobo rotation STOP |  | 254..255 | FD..FF | 99..100 |
| 26) Gobo 2 rot fine | Gobo 2 rot. Low | (0% - 100%) | 0..255 | 0..FF | 0..100 |
| 27) Effect wheel swing | Stepless swing of the "Motion Wheel" | 0 - 100% | 0..255 | 00..FF | 0..100 |
|  | Rotating stop |  | 0..1 | 00..01 | 00..1 |
| 28) Effect wheel | Rotating slow - fast CW |  | 1..128 | 01..80 | 1..50 |
| Posi./Rot | Rotating fast - slow CCW |  | 129...254 | 81..FD | 51..99 |
|  | Rotating stop |  | 254..255 | FD..FF | 99..100 |
| 29) Prism | Prism swing out |  | 0..5 | 00..05 | 0..2 |
|  | Prism position 0 ... 540° |  | 6..127 | 06..7F | 0..50 |
|  | Prism rotation stop |  | 128..129 | 80..81 | 50,1 |
|  | Prism rotation, slow-fast, CW |  | 130..192 | 82..C0 | 51..75 |
|  | Prism rotation, fast-slow, CCW |  | 193..253 | C1..FD | 76..99 |
|  | Prism rotation stop |  | 254..255 | FD..FF | 99.100 |
| 30) Iris coarse | Iris open – closed |  | 0..131 | 00..83 | 00..52 |
| 31) Iris fine | Iris low |  | 0..255 | 00..FF | 00..100 |

Locking and unlocking the Control Panel

The Display will auto look automatically if the Touch Panel will not use for 10s. To unlook the Touch Panel you have to press one of the menu buttons for 2s..

1) Colors

The predefined colors can be used as start colors for the Rainbow effect.  First select a desired start color, then activate the rainbow effect. All Impression Spot One will then begin from that color and execute the rainbow effect synchronously. Different Impression Spot One can have different start colors but will still execute the rainbow effect synchronously. If you choose a color different from the ones marked with 1) in the tables above the rainbow start-color will be red.

2) Rainbow-effect Stop

will pause this function. After resuming the rainbow-effect will be continued with the current color.

3) The Rainbow-effect

will run synchronously only if it is started from one of the predefined colors (see also 1) before).

4) Attention:

Please note that shutter frequencies over 10 Hz are prohibited in some countries. Especially frequencies in the range of 5 - 12 Hz can possibly cause epileptic seizures  continuous blink irritation with a latency period of min. 70 ms. Please also refer to the relevant legislation of each country (For Germany for example: BGI 810-4 (Anhang 2) "Sicherheit bei Produktionen und Veranstaltungen  Scheinwerfer").

5) Color Temperature

At anytime the IMPRESSION Spot One can be used the color correction for every RGB color.